[Description]

The updated version of HDCU4300 is released. (V2.12 => V2.31)
Upgrade the software in the unit as necessary.
In case of upgrading, upgrade the firmware and the PLD on each board by referring to the procedure
below. Also, upgrade the peripheral unit as necessary by referring to the related technical memo
information.

- ■ Related technical memo

- ・ HDCM18-066 (Part change for HKCU-4001)

- ・ HDCM18-078 (Operation explanation of LIVE TONE CONTROL)

- ・ HDCM19-032 (Change of HDCU4300 rear FAN bracket)

- ・ HDCM20-019 (Supplement for SR Live Meta and HDR-Look)

[When HKCU-IP43F is used]

- ・HDCM16-065  Software Release(LSM plig-in for HDCU4300, V1.1.3)

[When HKCU-4001 is used]

・HDCM20-061  Software Release [Device Setting Plug-in for HDCU4300-ST2110（LSM）,  V1.1.2]

- Recommended software version for related models and related technical memo

| - Technical memo No. | - Related models | - Version | - Technical memo |
| --- | --- | --- | --- |
| - HDCM20-055 | - HDC4300 | - V1.80 | - Software Release [V1.80] |
| - HDCM20-059 | - HDC-P43 | - V1.80 | - Software Release [V1.80] |
| - HDCM20-033 | - MSU-1000/1500 | - V3.60 | - Software Release [V3.60] |
| - HDCM20-032 | - RCP-1500/1501/1530 | - V3.60 | - Software Release [V3.60] |
| - HDCM20-034R | - RCP-3500/3501 | - V1.11 | - Software Release [V1.11] |
| - IPTM19-002 | - PWS-110NM1/100NM1 | - V2.2.0 | - Software Release [IP Live System Manager V2.2.0] |

- [Target serial]

| Model | Serial |
| --- | --- |
| HDCU4300 SY3 | 0030001~33068 |
| HDCU4300 SYL | 0010001~13539 |

[Change Point]

- ＜New Function＞

- 1．Add Format（Installed HKCU-4001）

- Add the following format  as IP OUTPUT.

- 1080/59.94P（2x）, 1080/59.94P（3x）, 1080/59.94P（4x）, 1080/59.94I（6x）

- 1080/50P（2x）1080/50P（3x）1080/50P（4x）1080/50I（6x）

2．Error notification to LMS

Added a function to check the error information of  HDCU on the LMS.

3.  Support SR Live Metadata（Refer to Technical Memo “HDCM20-019”）

Release Supplement.

4.  Support HDR Look（Refer to Technical Memo “HDCM20-019”）

Release Supplement.

5. Support  HDR Color Bar out

・ Changed to be able to set BARS TYPE separtely for HDR and SDR

・ Rename BAR 16:9(100%) to SDR-LOOK BAR 16:9(100%), and BAR 16:9(75%) to

SDR-LOOK BAR 16:9(75%)

・ Added HDR BAR described in ITU-R BT.2111 to HDR BAR TYPE

6.  Changed to reflect not only the light reception level but also the occurrence status of transmission failure

on the optical condition bar

The following warning is displayed on the Panel.

「CCU : Reflection Care」

When a warning occurs, it is recommended to clean the optical fiber as well as when the light level is attenuated.

7．Support Genlock mode Network on the Panel（Installed HKCU-4001）

When HKCU-4001 is installed, Genlock Mode SD / HD selection and Network selection on the panel are added.

<Error Correction>

1．[V2.12->V2.31]Fixed HD Prompter being disable even though it was enabled

2．[V2.12->V2.31]Fixed to be able to allocate 4K meta data stream on LSM

3．[V2.30->V2.31] Fixed the problem of not being able to connect to the Master unit when set to MCS in CNS Mode.

< Specification change>

1.	Adaptive Matrix extensions

The Level adjustment range of Adaptive Matrix has been expanded to Level = 0-14.

Please use in combination with HDC4300 V1.70 or higher and HDC-P43 V1.40 or higher.

2. Modification of VPID

Modify VPID for S-Log3 / S-Log2 output and 1080p

3. Added Enable / Disable setting for ST 2110 4K Video Stream　[Installed HKCU-4001]

4. Added Enable / Disable setting for SAP Announce　[Installed HKCU-4001]

[Preparation]

＜Update data files＞

File name： hdcu4300_v2.31.zip

* Obtain it from Software Download site.

The files below can be obtained by extracting hdcu4300_v2.31.zip.

| Package name |  | Package File Name | File Type |
| --- | --- | --- | --- |
| hdcu4300_v2.31_allpackage.zip | hdcu4300_v2.31.zip | hdcu4300_app.pkg | Software file |
|  |  | hdcu4300_mif.pkg | PLD file |
|  |  | hdcu4300_vif.pkg | PLD file |
|  |  | hdcu4300_4kpost.pkg | PLD file |
|  |  | hdcu4300_sdp.pkg | PLD file |
|  |  | hdcu4300_4001net1.pkg | PLD file |
|  |  | hdcu4300_4001net2.pkg | PLD file |
|  |  | hdcu4300_os.pkg | Software file |
|  |  | hdcu4300_updater.pkg | Software file |
|  |  | hdcu4300_sy.pkg | PLD file |
|  |  | hdcu4300_tx.pkg | PLD file |
|  |  | hdcu4300_dec_hdc4300.pkg | PLD file |
|  |  | hdcu4300_dpr.pkg | PLD file |
|  |  | hdcu4300_dpr_hco.pkg | PLD file |
|  |  | hdcu4300_dpr_sub.pkg | PLD file |
|  |  | hdcu4300_net.pkg | PLD file |

<Others>

USB memory x1

- [Upgrading Procedure]

- Upgrade the software using a USB memory.

- To upgrade to this version, place the all package above to the USB memory, and be sure to perform

- the procedure of *Forced version update*.

- As for the detail procedure, refer to SERVICE MANUAL 1-10.

- [Notes]

- When upgrading each package from service menu, it is possible that the following phenomena occur.

- ・ The menu is not displayed.

- ・ The version of NET1 / NET2 FPGA for HKCU-4001 is not displayed.

- Even in this case, it is possible to restore the correct state by performing “Forced version update”.

[Confirmation after Version Update]

Display the CCU MENU - <DIAGNOSIS> - <VERSION1> (D03) page and <VERSION2>(D04)

page, and check that the versions of the updated software and PLD are as shown in the table below.

Table 1 : Updated versions

| Item | Version |
| --- | --- |
| APPLICATION | V2.31 |
| OS | V2.11 |
| UPDATER | V1.10 |
| MIF | V2.10 |
| VIF | V2.20 |
| TX | V3.20 |
| SY | V1.01 |
| 4K-POST | V1.51 |
| SDP | V2.10 |
| DEC(HDC4300) | V1.10 |
| DPR(NORMAL) | V1.10 |
| DPR(HCO) | V2.00 |
| DPR(Sub) | V1.10 *1 |
| NET | V1.03 *2 |
| NMI LSI | V2.1.1_08 *2 |
| NET1 | V2.24 *3 |
| NET2 | V2.26 *3 |

*1 When DPR-392 board is installed

*2 When HKCU-IP43F is installed

*3 When HKCU-4001 is installed

[Supplement]

There is a limitation below by combining with CNU-700.

When CCU Format is 4K and 4K DTL of the Detail selection of the panel is not selected, the adjusted

data for the camera is not reflected to the DATA page of CNU-700.

KNEE, APT, DTL LEVEL, DTL LIMITER, DTL CRISP

[Software History]

|  | - V1.03 - 初回リリース | - V1.04 - HDCM - 16-012 | - V1.05 - HDCM - 16-020 | - V1.11 - HDCM - 16-030 | - V1.21 - HDCM - 16-076 | - V1.22 - HDCM - 17-010 | - V1.30 - HDCM - 17-036 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - APPLICATION | - V1.03 | - V1.04 | - V1.05 | - V1.11 | - V1.21 | - V1.22 | - V1.30 |
| - OS | - V1.00 | - ← | - ← | - V1.10 | - ← | - ← | - ← |
| - UPDATER | - V1.00 | - ← | - ← | - V1.10 | - ← | - ← | - ← |
| - MIF | - V1.00 | - ← | - ← | - V1.10 | - ← | - V1.11 | - ← |
| - VIF | - V1.03 | - ← | - ← | - V1.11 | - V1.12 | - V1.13 | - V1.14 |
| - TX | - V3.00 | - ← | - ← | - ← | - V3.11 | - ← | - V3.20 |
| - SY | - V1.00 | - ← | - ← | - ← | - ← | - ← | - ← |
| - 4K-POST | - V1.00 | - ← | - ← | - V1.10 | - V1.23 | - ← | - ← |
| - SDP | - V1.00 | - ← | - ← | - V1.10 | - V1.21 | - ← | - ← |
| - DEC(HDC4300) | - V1.00 | - ← | - ← | - ← | - ← | - ← | - ← |
| - DPR | - V1.00 | - ← | - ← | - ← | - ← | - ← | - ← |
| - DPR(HCO) | - -- | - -- | - -- | - -- | - V2.00 | - ← | - ← |
| - DPR(Sub) | - -- | - -- | - -- | - -- | - -- | - -- | - -- |
| - NET | - -- | - -- | - -- | - V1.00 | - V1.02 | - ← | - ← |
| - NMI LSI | - -- | - -- | - -- | - -- | - V1.4.1_02 | - ← | - V1.4.2_06 |
| - NET1 | - -- | - -- | - -- | - -- | - -- | - -- | - -- |
| - NET2 | - -- | - -- | - -- | - -- | - -- | - -- | - -- |

|  | - V1.40 - HDCM - 17-081 | - V1.52 - HDCM - 18-012 | - V1.60 - HDCM - 18-040 | - V2.03 - HDCM - 18-067 | - V2.11 - HDCM - 19-030 | - V2.12 - HDCM - 19-057 |
| --- | --- | --- | --- | --- | --- | --- |
| - APPLICATION | - V1.40 | - V1.52 | - V1.60 | - V2.03 | - V2.11 | - V2.12 |
| - OS | - ← | - ← | - ← | - V2.00 | - V2.11 | - ← |
| - UPDATER | - ← | - ← | - ← | - ← | - ← | - ← |
| - MIF | - ← | - V1.20 | - ← | - V2.00 | - V2.02 | - V2.03 |
| - VIF | - V1.20 | - ← | - ← | - V2.00 | - ← | - ← |
| - TX | - ← | - ← | - ← | - ← | - ← | - ← |
| - SY | - ← | - ← | - V1.01 | - ← | - ← | - ← |
| - 4K-POST | - V1.30 | - V1.32 | - ← | - ← | - ← | - ← |
| - SDP | - ← | - ← | - ← | - V2.02 | - ← | - ← |
| - DEC(HDC4300) | - ← | - ← | - ← | - ← | - V1.10 | - ← |
| - DPR | - ← | - ← | - ← | - ← | - V1.10 | - ← |
| - DPR(HCO) | - ← | - ← |  | - ← | - ← | - ← |
| - DPR(Sub) | - -- | - -- | - V1.00 | - ← | - V1.10 | - ← |
| - NET | - ← | - ← | - ← | - ← | - ← | - V1.03 |
| - NMI LSI | - V1.4.2_07 | - ← | - ← | - ← | - V2.1.1_08 | - ← |
| - NET1 | - -- | - -- | - -- | - V1.00 | - V2.04 | - ← |
| - NET2 | - -- | - -- | - -- | - V1.20 | - V2.04 | - ← |

|  | - V2.30 - HDCM - 20-056 - Canceled | - V2.31 - HDCM - 21-003 |
| --- | --- | --- |
| - APPLICATION | - V2.30 | - V2.31 |
| - OS | - ← | - ← |
| - UPDATER | - ← | - ← |
| - MIF | - V2.10 | - V2.10 |
| - VIF | - V2.20 | - V2.20 |
| - TX | - ← | - ← |
| - SY | - ← | - ← |
| - 4K-POST | - V1.51 | - V1.51 |
| - SDP | - V2.10 | - V2.10 |
| - DEC(HDC4300) | - ← | - ← |
| - DPR | - ← | - ← |
| - DPR(HCO) | - ← | - ← |
| - DPR(Sub) | - -- | - -- |
| - NET | - ← | - ← |
| - NMI LSI | - ← | - ← |
| - NET1 | - V2.24 | - V2.24 |
| - NET2 | - V2.26 | - V2.26 |