[Description]

Updated version of the software for HZC-CSM10 is released. (V1.3.0 -> V1.4.0)

Upgrade it as necessary.

[Change Points]

<New Functions>

- Added Overview(Absolute) item (Rename tab to “HDR-SDR Relation Item”)

- Added HFR Detail Ratio adjustment function

- Added ARIA Gain Limit setting

- Added Virtual IRIS setting

- Added Remote Back Focus adjustment

- Supports 4K Return setting and added OETF/Look/Color Space setting in Return page

- Added Look setting in Multi Format page

- Added Soft Knee and Knee Max setting

- Added Network in Genlock mode setting page

- Added the setting to detect SR Live Meta of HDRC-4000

<Functional Improvement>

- Improved display of Cable Condition

<Detail of new functions>

- Added Overview(Absolute) item (Rename tab to “HDR-SDR Relation Item”)

For SR Live operation, added an item in the “Overview(Absolute)” of V1.3.0 and changed the tab name to “HDR-SDR Relation Items”.

The following are newly added items.

| HDR Operation |  | HDR Black Compression On |
| --- | --- | --- |
|  | HDR Setup | SDR Gain |
|  |  | HDR Black Offset |
|  | HDR Knee | HDR Knee On |
|  |  | HDR Knee Point |
|  |  | HDR Knee Slope |
|  | HDR White Clip | HDR White Clip On |
|  |  | HDR White Clip Master |
|  | HDR Black Clip | HDR Black Clip On |
| Gamma Table |  | Gamma Type |
|  |  | Standard |
|  |  | Hyper |
|  |  | Special |
|  |  | User |
| Knee | Soft Knee | Soft Knee On |
|  |  | Soft Knee Radius |
| White Clip |  | While Clip On |
|  |  | White Clips Master |

- Added HFR Detail Ratio adjustment function

Control Panel > Paint tab

- Added ARIA Gain Limit setting

Control Panel  > Maintenance > Lens

- Added Virtual IRIS setting

Control Panel > Maintenance > Lens

- Added Remote Back Focus adjustment

Control Panel > Maintenance > Lens

[NOTE]

The default for this setting is hidden. Displayed only when the “Back Focus” check box is checked 
in Tools >Configuration >Permission tab > Item permission.

- Supports 4K Return setting and added OETF/Look/Color Space setting in Return page

Return Setting of input settings supports 4K Format, and Added OETF/Look/Color Space setting in
Control Panel > Configuration > CCU > Return Settings

- Added Look setting in Multi Format page

Control Panel > Configuration > Multi Format

- Added Soft Knee and Knee Max setting

Added “Soft Knee” setting in Knee and added “Knee Max” in Knee Point.

Control Panel > System Manage > Overview tab > Knee

Control Panel > Paint > Knee

- Added Network in Genlock mode setting page

Added Network in Genlock Mode options.

Control Panel > Configuration > CCU > Mode

- Added the setting to detect SR Live Meta of HDRC-4000

Added “SR Live Metadata” setting in Control Panel > Configuration > Converter.

<Detail of functional improvement>

- Improved status display of Cable Condition

The bar of Optical Condition has been changed to reflect not only the light reception level but also the occurrence status of transmission failures. If there is a transmission failure that seems to be affected by the light reflection at the optical fiber junction, the words “Reflection” and “Refl” are displayed on the the bar status display of Cable Condition.

[Preparation]

<Software Files>

Filename:

HZC-CSM10_1.4.0_Windows7.zip (For Windows7)

HZC-CSM10_1.4.0_Windows8.1_Windows10.zip (For Windows8.1/Windows10)

* Obtain the files from eCSite.

[Installation Procedure]

<Condition>

New installation or in state that the V1.3.0 of HZC-CSM10 is installed, the installation is possible in

both cases. Or, after uninstalling the previous version, the new installation of V1.4.0 is also possible.

<Procedure>

Use the installer depending on the OS of PC for installation.

The installation key of V1.4.0 is common for the conventional version.

When uninstalling the old version, the installation key is deleted. Before uninstallation, confirm the

version and the installation key by menu bar Help => “About Camera System Management Software”.

As for how to use the installer, refer to the file of “ReadThisFirst”.

[Confirmation / Adjustment]

Confirm if the version is V1.4.0. As for the version, confirm it by menu bar Help => “About Camera

System Management Software”.