# Changing the BDX-M1000 IP Address

Preface

The BDX-M1000 has 5 Ethernet (10BASE-T) ports on its rear panel.  Control of the unit is via port #5 ONLY.  Do not connect to the other ports, please.  Read this procedure over several times BEFORE you attempt the operation.  Make sure you know where all of the switches referred to are located.

The following procedure will set the IP address for port #5 only.  Please refer to RFC 1918 for guidance is selecting IP addresses for use in your network.  This equipment is NOT intended to be used on an open network with non-compression system equipment (such as personal computers or printers) whether or not the network is connected to the Internet.

If necessary, get the assistance of a trained network administrator skilled in configuring and maintaining TCP/IP networks.

Procedure

Open the front panel of the multiplexer.  Ensure that all 4 sets of front mounted DIP switches on the SY board are all down, with the exception of S102-2, which should be UP (this is the left-most DIP switch of the 4).

Connect your computer’s serial port to the 9 pin connector labeled “RS232C” on the right lower rear of the multiplexer.  Start a terminal emulator program.  Set the datacomm parameters for 9600, E, 8, 1.  It would be safest to also begin logging the session to disk.  The method for doing this will vary with the emulator program in use.  Logging to disk will permit you to recover if there is some sort of major problem, since you will be able to restore any settings inadvertently damaged.

The following procedure may seem initially like it requires 3 hands.  It is easily done with two, but take your time and use care to ensure you are pressing the correct switches.

Press S101 (labeled FUNCTION) AND S802 (CPU#1 RESET) simultaneously.  These two switches are located at the left end of the LED row D103 through D107.  S101 is located slightly behind D103.  S802 is located just in front of S101.  Now turn on the multiplexer power or reset the multiplexer via the S803 “System Reset” switch at the left front of the SY board if it is already powered up.

After LED’s begin to flash, release S802 (CPU#1 RESET) while continuing to hold down S101 (FUNCTION).  After several seconds, the red LED’s D107 and D108 will start blinking.  Now release S101.

You should get the following displayed on the terminal emulator:

*****  BDX-M1000 Service Mode  *****

-- Firmware Information (in on-board FLASH ROM) --

*** Normal mode Application ***

Ver.0.91 98/08/24

*** Standalone mode Application ***

Ver.0.29

BDX-M1000 Standalone Mode(SI-CPU)

Created : 1998/07/30

*** Board Support Package ***

Ver.1.12

BDX-M1000 SY-244 BSP

Created : 1998/08/20

Required ROM size = 961664(0x0EAC80)bytes

-- Service Menu --

1 : Write Initial Hardware Parameter to EEPROM

2 : Modify Network Parameter in EEPROM

Select operation ->  [2]

Select operation number 2:

Select operation ->  [2] 2<Enter>

You will now get a series of messages that show current parameters and permit you to make changes.  Carefully hit <Enter> for each one until you reach the LAN#5 IP address.  The following is an illustration of what you will see (the changed entry is shown below underlined for clarity):

Serial Number (EEPROM ID) = 10044

*** CPU#3 AUX-CPU ***

LAN#1 Parameter...

IP Address   [192.168.3.1]

Subnet Mask  [255.255.255.0]

Default Gateway (None=0.0.0.0)  [0.0.0.0]

*** CPU#2 CA-CPU ***

LAN#2 Parameter...

IP Address   [192.168.4.2]

Subnet Mask  [255.255.255.0]

LAN#3 Parameter...

IP Address   [192.168.3.2]

Subnet Mask  [255.255.255.0]

Default Gateway (None=0.0.0.0)  [0.0.0.0]

*** CPU#1 SI-CPU ***

LAN#4 Parameter...

IP Address   [192.168.4.3]

Subnet Mask  [255.255.255.0]

LAN#5 Parameter...

IP Address   [192.168.3.3] 192.168.3.10

Subnet Mask  [255.255.255.0]

Default Gateway (None=0.0.0.0)  [0.0.0.0]

It will then display the results of any changes all at once:

Serial Number   10044

*** CPU#3 AUX-CPU ***

LAN#1 Parameter...

Ethernet(MAC) Address  8:0:46:1:8E:97

IP Address  192.168.3.1

Subnet Mask 255.255.255.0

Default Gateway (None=0.0.0.0)  0.0.0.0

*** CPU#2 CA-CPU ***

LAN#2 Parameter...

Ethernet(MAC) Address  8:0:46:1:8E:98

IP Address  192.168.4.2

Subnet Mask 255.255.255.0

LAN#3 Parameter...

Ethernet(MAC) Address  8:0:46:1:8E:99

IP Address  192.168.3.2

Subnet Mask 255.255.255.0

Default Gateway (None=0.0.0.0)  0.0.0.0

*** CPU#1 SI-CPU ***

LAN#4 Parameter...

Ethernet(MAC) Address  8:0:46:1:8E:9A

IP Address  192.168.4.3

Subnet Mask 255.255.255.0

LAN#5 Parameter...

Ethernet(MAC) Address  8:0:46:1:8E:9B

IP Address  192.168.3.10

Subnet Mask 255.255.255.0

Default Gateway (None=0.0.0.0)  0.0.0.0

Write EEPROM OK? (y/n)  [n]

Following this last entry, you will be asked if you wish to save the settings to EEPROM.  If you have any concern about the value(s) you have just entered, answer ‘N”.  You may scroll back and examine the values displayed if your terminal emulator permits this.

Write EEPROM OK? (y/n)  [n] y

Writing ..... Done

The last message shows the status of the write and when it is complete.

Remember, the values in brackets are the current values for that field.  Just press <Enter> to leave each of these intact.  You should only enter a value for LAN#5 IP address.  Do not enter the same number as is assigned to another port!  Double check your work before answering ‘Y’ to write into EEPROM.

If there is anything amiss, start from the beginning and redo the entry.

DO NOT enter any Default Gateway information unless your network is large enough to include routers which map one subnet to another.  USE EXTREME CAUTION in entering this information.  A mistake will make the ports malfunction.

Please also note that “MAC Addresses” are displayed for your information only, there is no ability to change them.

If the subject of network administration makes you panic, there is a very understandable book on the subject titled TCP/IP Network Administration by Craig Hunt (ISBN 0-937175-82-X.  Reading this may easy your anxiety, but as stated earlier, you may wish to get professional assistance in setting up your data network.