# UCP VERSION UPGRADE PROCEDURE

CHECKING CURRENT VERSION IN UCP PANEL

(This procedure must be done using a serial connection to the diagnostic port on the UCP panel)

- Connect the 9 pin serial port on the PC running TeraTerm to the RJ-45 to 9 pin adapter (9 pin side) provided by Utah Scientific (labeled SC4/2020 ). NOTE: if this is a laptop from Utah Scientific then connect to the 9 pin serial port on the USB to serial cable provided with the laptop.

- Connect the UCP diagnostic serial port to the RJ-45 to 9 pin adapter (RJ45 side) provided by Utah Scientific (labeled SC4/2020) using a standard straight through CAT 5 cable.

- Launch Tera Term Pro and open the Setup menu and select ‘Serial Port’.

- Select the computer COMM port that is being used.

- Set the baud rate to 19200.

- Set the Data to 8 bit.

- Set the Parity to None.

- Set the Stop to 1 bit.

- Set Flow Control to None.

- Click OK.

- At the prompt you should see the Node number of the panel you are connected to.

- Type the letter V and then press 'Return' to see the current version.

- Close Tera Term before performing the upgrade.

UPGRADE PROCEDURE

(Use this procedure for all UCP panel types)

- Navigate to the folder called release-ucp found on the system CD and open it.

- Double click the setup.bat file. This will create a new folder called ucp located in C:\usi.

- Open the new folder from step 2 and then open the folder with the highest version number. (Example: v1.93)

- Make sure there are no sessions of TeraTerm running at this time and then open the Unic807 folder and double click the UcpUpgrade.bat file.  NOTE:  The only time the Unic803 folder is to be used is if there are UCP-64 style control panels that need to be upgraded.  In this case refer to the next section for the procedure.

- The upgrade will run automatically from here and prompt you that it is done.

- Use the procedure above to check that the new version has been loaded successfully.

UPGRADE PROCEDURE FOR UCP-64 STYLE PANELS WITH UNIC803 CHIP SET

(Use this procedure only for UCP-64 panel types)

- Use the procedure above for checking the version type.

- After typing the letter V you will see the version number and at the end of that line it will either have an 803 or an 807 which is the chip type.

- If it is an 803 then follow the same upgrade procedure found above only in step 4 open the Unic803 folder and proceed from there.