Partner Phone Programming

- “Feature-0-0”

-“Intercom, Intercom” (the left intercom button)

This puts you in “System Programming”, once there, use the following:

*note – to exit, simply pick up and hang up the handset

# 101    System Date    Set system date

# 102    System Day    Set system day

# 103    System Time    Set system time

# 104    Number of Lines    Set numbers of trunk lines

# 105    Transfer Return Rings    Transferred call return to    orginator

# 107    Recall Timer Durations    Time is ms before recall starts

# 108    Rotary Dialing Timeout    Set only for T/R device on a rotary line

# 109    Outside Conference Denial    Allow = 2 outside caller / 3 inside caller

Disallow = 1 outside caller / 4 inside caller

# 110    ASA Delay - automatic system answer    #'s of rings at extension 10 before ASA

# 111    ASA Button - on LED button only    Program a button to turn ASA on / off

# 112    DXD Delay - direct extension dial    #'s of rings at extension 10 before DXD

# 113    DXD Button - on LED button only    Program a button to turn DXD on / off

# 114    Outside Call Restriction Button - LED    Set call restrictions on extension 10

Must program extensions on buttons in

order to work

# 115    Wake Up Service Button - LED    Set wake up call service on extension 10

Must program extensions on buttons in

order to work

# 116 / # 320    Call Coverage Rings    #'s of rings before coverage extension takes over

# 117 / # 321    VMS Cover Rings    #'s of rings before voice mail takes over

# 119    Ring On Transfer    User hear rings if turned active, must turn off if music

on hold is in use

# 121    ASA Mode    HOLD, DISCONNECT, RING

# 122    Caller ID Type    USA or Singapore

# 123    Backup Programming Automatically    Every 1st day of the month at 2:00 am

# 124    Backup Programming Manually    Manual backup process

# 125    Restore Programming    Restore either auto or manual backup program

# 126    Automatic Daylight / Standard Times    Set auto daylight time adjustment on system

# 198    Pool Mode - 8632852    Pool mode

# 198    Key Mode - 8632851    Key mode

# 201    Dial Mode    Set line dial mode to touch tone or rotary

# 203    Hold Disconnect Time    Time to disconnect if put on hold caller disconnects

# 204    ASA Lines    Assign lines to ASA

# 205    DXD Lines    Assign lines to DXD

# 206    Group Call Distribution - AA    Assign lines or hunt group to groups

# 207    Pool Line Assignment    Add lines to pool according to it's pool number

# 208    Line Coverage Extension    Set a backup coverage extension

# 209    Unique Line Ringing    Assign different ring tone to lines

# 301    Line Assignment    Assign lines to extensions

# 302    Line Access Restriction    Set line access restrictions to extensions

# 303    Display Language    Set display languages to extensions

# 304    Automatic Extension Privacy    Prevent other user to intercept a call

# 305    Abbreviated Ringing    Set numbers of ring on a second incoming line

while on a phone call

# 306    Transfer Return Extension    Set extension where calls should return

# 307    Forced Account Code Entry    Set to force account code entry before making a call

# 308    Distinctive Ring    Set to have calls to ring differently depends on the source

# 309    Intercom Dial Tone    Set intercom dial tone sounds like a CO dial tone

# 310    Automatic VMS Cover    Route unanswer calls to VM

# 311    External Hotline    Set a specific extension to be a hotline extension.  Must be a

single line phone, program outgoing phone number into speed

dial list ( Feature 80 )

# 312    Voice Interrupt on Busy    Interrupt a call, program this feature and assign interrupe on busy

talk back button.  Usage: INTERCOM *ext#

# 313    Line Access Mode    Change extensions from Pool to Key or Key to Pool.  All previously

programmed lines will be removed

# 314    Pool Extension Assignment    Assign pools to extensions

# 315    Pool Access Restriction    Set dialing permission to pool extensions

# 316    Call Waiting    To activate call waiting feature from the Partner system

# 317    Caller ID Log Answered Calls    Log only answered calls at each extension

# 318    Caller ID Log Line Assocation    Associate 1 extension to log all calls

# 319    Caller ID Log All Calls    Log all calls at each extension

# 399    Copy Settings    Copy a certain limit of features to another set of phones

# 401    Outgoing Call Restriction    Set call restrictions on lines assigned to extensions

# 402    Toll Call Prefix    Set if 0 or 1 is not require for long distance

# 403    System Password    Root password to override dialing restrictions and turn on/off night

service button

# 404    Disallowed Phone Number Lists    Set a list of restricted outgoing phone numbers

# 405    Disallowed List Assignments    Assign disallow list to extensions

# 406    Emergency Phone Number Lists    Numbers regardless of dialing restrictions unless forced account code

is in effect

# 407    Allowed Phone Number Lists    Set a list of allowed phone numbers regardless of dialing restrictions

# 408    Allowed List Assignments    Assign allow list to extensions

# 409    Forced Account Code List    Set a list of account codes of up to 99 account codes

# 410    Star Code Dial Delay    Set a certain time delay after the star code entry

# 501    Pickup Group Extension    Allow others in the pickup group to pick up a call, assign a pickup button

or pickup manually = 66# - group number

# 502    Calling Group Extension    Assign extensions to different groups, allow group paging or ringing

# 503    Night Service Button    Set a button to turn night service on/off

# 504    Night Service Group Extensions    Assign extensions to night service group when the service is turn on

# 505    Hunt Group Extension    Assign extensions to different hunt groups, use group call distribution

to assign lines to groups

# 506    VMS Hunt Delay    Determine how VMS takes over a call after a certain numbers of rings

# 507    VMS Hunt Schedule    Set how VMS handle calls during the day

# 601    Fax Machine Extension    Assign fax extensions, set a fax management button for easy transfer

# 602    Music On Hold    Turn music on hold on or off

# 603    Hotline    Set an internal hotline using the intercom feature only

# 604    Doorphone 1 Extension    Set doorphone extension 1

# 605    Doorphone 2 Extension    Set doorphone extension 2

# 606    Doorphone Alert Extension    Set doorphone alert extension

# 607    Automated Attendant Extension    Set AA extension to allow transfer return, do not use in conjuction with

the VMS' AA

# 608    SMDR Record Type    Set how SMDR should record calls

# 609    SMDR Top of Page    Notify the system after a printer reset or paper problem

# 610    SMDR Output Format    Set the output format, either 15 or 24 digits

# 611    SMDR Talk Time    Records also the length of the call

# 612    Contact Closure Group    Assign extensions to operate contact closure devices

# 613    Contact Closure Operation Type    Set how contact closure operator operates the devices

# 614    Music On Hold Volume    Set volume for music on hold

# 728    System Reset - SAVED    System reset, programming saved

# 730    Remote Administration Password    Set remote password

# 989    System Reset - NOT SAVED    Master reset - password = CLEAR

Button or Dial Code Programming

F 01    Do Not Disturb - LED

F 02    Exclusive Hold

F 03    Recall

F 04    Save Number Redial

F 05    Last Number Redial

F 06    Conference Drop

F 07    Privacy - LED

F 08    Touch Tone Enable

F 09    Message Light On

F 10    Message Light Off

F 11    Call Forwarding - LED

F 12    Account Code Entry - LED

F 14    VMS Transfer

F 15    VMS Cover - LED

F 16    Caller ID Name Display - LED

F 17    Caller ID Inspect - LED

F 18    Voice Interrupt on Busy Talk Back - LED

F 19    Background Music - LED

F 20    Call Coverage - LED

F 21    Station Lock - LED

F 22    Station Unlock

F 23    Caller ID Logging and Dialing

F 25    Call Screening

F 41    Contact Closure 1 - LED

F 42    Contact Closure 2 - LED

F 59    System Release Status